LIFE SAFETY & FIRE INSPECTION CHECKLIST

College Classroom & Lecture Buildings

Y = Meets Requirements; N= Does Not Meet Requirements

Y   N                                                 		       Y   N

EXTERIOR:

	Fire Lanes marked/unobstructed (MSFC 503.3, 503.4)

	Address visible (MSFC 505.1)

	Fire hydrants/water supply accessible (MSFC 507)

	Combustible accumulations/storage acceptable

(MSFC 304.2, MSFC 315.4)

	F.D. key box present/maintained (MSFC 506)

	F.D. connections visible/maintained (MSFC 912)

	Gas meter/piping protected against impact

(MSFC 312, 603.9)

	Dumpsters are outside and 5 ft. or more from

combustible walls, openings or combustible roof eave

lines. (MSFC 304.3.3)

EXIT FEATURES/MEANS OF EGRESS:

	Adequate number of exits

(MSFC 1004.2, 1006, 1016, 1104.25)

	Exits, aisles, & corridors free of obstructions

(MSFC 1031.3)

	Exit doors open from inside (MSFC 1010.1.9)

	Exit signs present and operational (MSFC 1013, 1104.3)

	Emergency lighting present, operational & tested

(MSFC 1008, 1104.5)

	Exit corridors rated (MSFC 1020.1, 1104.17)

	No locks, chains, keys or special knowledge required to

open exit doors (MSFC 1010.1.9.4)

	Panic hardware on exit doors if capacity of room is 50 or

more people for assembly use (MSFC 1010.1.10)

CONSTRUCTION FEATURES:

	Fire rated construction present/maintained

(MSFC 703.1)

	Incidental use areas separated/sprinklered

(MSFC 1105.3)

	Rated/required fire doors operational (MSFC 704.2)

	Vertical openings protected (MSFC 1103.4)

STORAGE:

	Storage neat and orderly (MSFC 315.3)

	Storage of combustible materials kept to a minimum (MSFC 304.2)

	Combustible storage prohibited in electrical equipment

rooms, restrictions apply in boiler/mechanical rooms

(MSFC 315.3.3)

	Storage maintained [2’ below ceiling un-sprinklered,

18” sprinklered] (MSFC 315.3.1)

	Rubbish/waste subject to spontaneous ignition shall be disposed of daily and in listed container (MSFC 304.3.1)

	Fuel-powered equipment shall be stored outdoors or in

fire rated room (MSFC 313.1)

FLAMMABLE/COMBUSTIBLE LIQUIDS:

	Safety containers if used (MSFC 5005.1.10, 5705.2.4)

	Adequate ventilation (MSFC 5705.3.7.5.1)

	No basement storage unless sprinklered

(MSFC 5704.3.5.1)

	No sources of ignition (MSFC 5704.2.4)

	Quantities >10 gals in cabinet (MSFC 5704.3.4.4)

	Not stored/kept near exits (MSFC 5704.3.3.3)

UTILITY/MECHANICAL/HVAC:

	Heating equipment serviced annually to ensure proper

operation (MSFC 603.5.2)

	Approved piping, connections & appliances used

(MSFC 603.1.2)

	Combustion air supply/venting present (MSFC 603.1)

	Adequate clearance between combustibles and appliances (MSFC 305.1, 603.5.3)

	Commercial hood ducts/filters are clean

(MSFC 607.3.3)

	Spray finishing to be conducted in separate booth or room

with adequate ventilation (MSFC 2404.2)

HAZARDOUS MATERIALS:

Compressed gas cylinders secured (MSFC 5303.5)

Rooms/areas properly labeled (MSFC 5003.5)

Acceptable quantities (MSFC 5003.1.1)

Separation of incompatible materials (MSFC 5003.9.8)

Safety Data Sheets available for inspection (MSFC 5003.4)

ELECTRICAL:

	No electrical hazards (MSFC 604.1, 604.6)

	Appliances/fixtures in good condition (MSFC 604.7)

	Power taps are correct design & properly used

(MSFC 604.4)

	Extension cords not substituted for fixed wiring

(MSFC 604.5)

	Access/clearance to electrical panels (MSFC 604.3)

	Electrical Rooms labeled (MSFC 604.3.1)

FIRE SAFETY PRECAUTIONS:

	Proper clearance between combustible materials and

heaters (MSFC 305.1)

	No open flames (lighters, matches, etc.) in classrooms/

lecture halls/offices unless a permit is obtained

(MSFC 308.3)

	Keep combustible rubbish to a minimum

(MSFC 304.1, 304.2)

	No hot work (welding, grinding) near combustible materials unless shielded (MSFC 3504.1.1)

	Maintain fire watch 30 minutes after hot work (welding, grinding) is finished (MSFC 3504.2.1)

	Fire safety and evacuation plans created and maintained

(MSFC 404)

FIRE PROTECTION SYSTEMS:

	Extinguishers, pull stations, sprinkler controls are readily

accessible (MSFC 901.6, 903.3.1, 906.5, 509)

	All systems tested/inspected at least annually

(MSFC 901.6.1, 904.12.5, 907.8)

	Coverage is adequate and properly installed

(MSFC 903.3.1, 905.2, 907, Chapter 11)

	Control panel/annunciator panel in normal condition

[no trouble or alarm conditions] (MSFC 901.6, 901.7)

	Commercial Kitchen Hood System fuel shutoff present (MSFC 904.12.2)

LIFE SAFETY & FIRE INSPECTION RECOMMENDATIONS

College Classroom & Lecture Buildings

NOTE: The information contained below are recommendations to ensure life and fire safety in the event of an emergency and not actual Minnesota State Fire Code (MSFC) requirements.

- Ensure employees know how to evacuate from work areas and perform any assigned duties in an emergency.

- Employees should know the location of the nearest means to activate the fire alarm system and extinguisher.

- In the event of a fire, occupants should only fight the fire IF:

  - Fire is not spreading rapidly

  - Fire is not blocking means of escape

  - Occupants are trained to use equipment needed to fight fire

If none of the above are possible, leave the area IMMEDIATELY

- Post evacuation routes in each classroom/lecture hall/auditorium located near exit doors

- If chemicals or hazardous materials are required, order and store the minimum amount required for teaching, instruction or use.

- Survey chemicals or hazardous materials at least annually and properly discard any that are expired or show signs of decomposition.

- Regularly clean lint traps on dryers and filters on vents, fans, air conditioners or other mechanical equipment

- Clean up spills of flammable liquids or other hazardous materials quickly and safely